Quality and Reliability Data

Last Updated on 5/30/2002

Atmel Serial EEPROM AT24C02A

Description		2K 2-Wire Shrink

Entry Date		2/25/2001

Tapeout Date		11/05/2001

Fab Out Date		2/11/2002

Char Date		1/25/2002

Date Qualified		5/30/2002

Release Engineer	Neal Scheibe

Sustaining Engineer	Neal Scheibe

#### Package Options

PACKAGE_OPTION		FIT/ROUTE		WAIVER	      	COMMENTS

EIAJ 8LD                                   YES                                 NO

PDIP 8LD 			YES			 NO

SOIC 8LD                                  YES                                 NO

TSSOP 14LD      		YES                        	 NO

DBGA                                         YES                                NO

LAP 5X6                                     YES                                NO

LAP 5X8                                     YES                                NO

TSSOP                                         YES                               NO

##### Voltage Options

AUTOMOTIVE	INDUSTRIAL     	COMMERCIAL

VOLTAGE		GRADE		GRADE	GRADE

5.00				PENDING		YES			YES

2.70				PENDING		YES			YES

1.80           			PENDING		YES			YES

Write Endurance

TRACK #	DATE	TEST_POINT	FAIL_QTY		COMMENTS

8355                  4/24/2002            1000000	0		NO FAILS

##### Life Test

TRACK #	DATE	TEST_POINT	FAIL_QTY		COMMENTS

8355                  4/24/2002           504                      		0            		NO FAILS

Data Retention

TRACK #	DATE 	RD_POINT           	FAIL_QTY		COMMENTS

8355     	4/24/2002 	 504                       	 		0 		NO FAILS

Characterization Data

Characterization Program

CH24C02A  Serial EEPROM  11-03-98

----------------------------------------

Opens:

Shorts:

Lkg @5.5V, #1 =  00.03      UA

Lkg @5.5V, #2 =  00.03      UA

Lkg @5.5V, #3 =  00.01      UA limit!

Lkg @5.5V, #5 =  00.09      UA

Lkg @5.5V, #6 =  00.01      UA limit!

Lkg @5.5V, #7 =  00.24      UA

ISB @ 06.000  VOLTS =  003.4      UA

ISB @ 05.500  VOLTS =  002.6      UA

ISB @ 05.000  VOLTS =  002.2      UA

ISB @ 04.500  VOLTS =  001.6      UA

ISB @ 04.000  VOLTS =  001.3      UA

ISB @ 03.500  VOLTS =  001.0      UA

ISB @ 03.000  VOLTS =  000.7      UA

ISB @ 02.500  VOLTS =  000.5      UA

ISB @ 02.000  VOLTS =  000.4      UA

ISB @ 01.500  VOLTS =  000.2      UA

Ack & Slaves:

Ack LVcc = 01.200  VOLTS limit!

WR-RD byte LVcc = 01.320  VOLTS

WR-RD array LVcc = 01.480  VOLTS

WR/RD MATRIX AT 100KHz:

5.5v 5.0v 4.5v 3.6v 2.7v 2.5v 1.8v 1.7v

---- ---- ---- ---- ---- ---- ---- ----

wrCHK @5.5v, rd:  P    P    P    P    P    P    P    P

wrICK @5.5v, rd:  P    P    P    P    P    P    P    P

wrCHK @5.0v, rd:  P    P    P    P    P    P    P    P

wrICK @5.0v, rd:  P    P    P    P    P    P    P    P

wrCHK @4.5v, rd:  P    P    P    P    P    P    P    P

wrICK @4.5v, rd:  P    P    P    P    P    P    P    P

wrCHK @3.6v, rd:  P    P    P    P    P    P    P    P

wrICK @3.6v, rd:  P    P    P    P    P    P    P    P

wrCHK @2.7v, rd:  P    P    P    P    P    P    P    P

wrICK @2.7v, rd:  P    P    P    P    P    P    P    P

wrCHK @2.5v, rd:  P    P    P    P    P    P    P    P

wrICK @2.5v, rd:  P    P    P    P    P    P    P    P

wrCHK @1.8v, rd:  P    P    P    P    P    P    P    P

wrICK @1.8v, rd:  P    P    P    P    P    P    P    P

wrCHK @1.7v, rd:  P    P    P    P    P    P    P    P

wrICK @1.7v, rd:  P    P    P    P    P    P    P    P

WR/RD MATRIX AT 400KHz:

5.5v 5.0v 4.5v 3.6v 2.7v 2.5v 1.8v 1.7v

---- ---- ---- ---- ---- ---- ---- ----

wrCHK @5.5v, rd:  P    P    P    P    P    P    P    P

wrICK @5.5v, rd:  P    P    P    P    P    P    P    P

wrCHK @5.0v, rd:  P    P    P    P    P    P    P    P

wrICK @5.0v, rd:  P    P    P    P    P    P    P    P

wrCHK @4.5v, rd:  P    P    P    P    P    P    P    P

wrICK @4.5v, rd:  P    P    P    P    P    P    P    P

wrCHK @3.6v, rd:  P    P    P    P    P    P    P    P

wrICK @3.6v, rd:  P    P    P    P    P    P    P    P

wrCHK @2.7v, rd:  P    P    P    P    P    P    P    P

wrICK @2.7v, rd:  P    P    P    P    P    P    P    P

wrCHK @2.5v, rd:  P    P    P    P    P    P    P    P

wrICK @2.5v, rd:  P    P    P    P    P    P    P    P

wrCHK @1.8v, rd:  P    P    P    P    P    P    P    P

wrICK @1.8v, rd:  P    P    P    P    P    P    P    P

wrCHK @1.7v, rd:  P    P    P    P    P    P    P    P

wrICK @1.7v, rd:  P    P    P    P    P    P    P    P

WR/RD MATRIX AT 1MHz:

5.5v 5.0v 4.5v 3.6v 2.7v 2.5v 1.8v 1.7v

---- ---- ---- ---- ---- ---- ---- ----

wrCHK @5.5v, rd:  P    P    P    P    P    F    F    F

wrICK @5.5v, rd:   P    P    P    P    P    F    F    F

wrCHK @5.0v, rd:   P    P    P    P    P    F    F    F

wrICK @5.0v, rd:   P    P    P    P    F    F    F    F

wrCHK @4.5v, rd:   P    P    P    P    P    F    F    F

wrICK @4.5v, rd:   P    P    P    P    P    F    F    F

wrCHK @3.6v, rd:   P    P    P    P    P    F    F    F

wrICK @3.6v, rd:   P    P    P    P    F    F    F    F

wrCHK @2.7v, rd:   P    P    P    P    P    F    F    F

wrICK @2.7v, rd:   P    P    P    P    P    F    F    F

wrCHK @2.5v, rd:   P    P    P    P    P    F    F    F

wrICK @2.5v, rd:   P    P    P    P    P    F    F    F

wrCHK @1.8v, rd:   F    F    F    F    F    F    F    F

wrICK @1.8v, rd:   P    P    P    P    P    F    F    F

wrCHK @1.7v, rd:   F    F    F    F    F    F    F    F

wrICK @1.7v, rd:   P    P    P    P    P    F    F    F

fSCL@100KHz: Vcc = 01.320  VOLTS

fSCL@200KHz: Vcc = 01.340  VOLTS

fSCL@300KHz: Vcc = 01.400  VOLTS

fSCL@400KHz: Vcc = 01.480  VOLTS

fSCL@500KHz: Vcc = 01.640  VOLTS

fSCL@600KHz: Vcc = 01.820  VOLTS

fSCL@700KHz: Vcc = 01.980  VOLTS

fSCL@800KHz: Vcc = 02.140  VOLTS

fSCL@900KHz: Vcc = 02.400  VOLTS

fSCL@1.0MHz: Vcc = 02.560  VOLTS

fSCL@1.1MHz: Vcc = 02.940  VOLTS

fSCL@1.2MHz: Vcc = 03.260  VOLTS

fSCL@1.3MHz: Vcc = 03.440  VOLTS

fSCL@1.4MHz: Vcc = 06.000  VOLTS limit!

fSCL@1.5MHz: Vcc = 01.200  VOLTS limit!

ICCrd @5.5v, 400KHz =  00.40      MA

ICCrd @5.0v, 400KHz =  00.30      MA

ICCrd @4.5v, 400KHz =  00.25      MA

ICCrd @3.6v, 100KHz =  00.05      MA limit!

ICCrd @2.7v, 100KHz =  00.10      MA

ICCrd @2.5v, 100KHz =  00.10      MA

ICCrd @1.8v, 100KHz =  00.05      MA limit!

ICCwr @5.5v, 100KHz =  02.10      MA

ICCwr @5.0v, 100KHz =  02.05      MA

ICCwr @4.5v, 100KHz =  01.80      MA

ICCwr @3.6v, 100KHz =  01.50      MA

ICCwr @2.7v, 100KHz =  01.25      MA

ICCwr @2.5v, 100KHz =  01.10      MA

ICCwr @1.8v, 100KHz =  00.50      MA

VIL\H @5.5v =  02.050  VOLTS,  02.750  VOLTS

VIL\H @5.0v =  01.900  VOLTS,  02.500  VOLTS

VIL\H @4.5v =  01.700  VOLTS,  02.300  VOLTS

VIL\H @3.6v =  01.450  VOLTS,  01.900  VOLTS

VIL\H @2.7v =  01.150  VOLTS,  01.500  VOLTS

VIL\H @2.5v =  00.000  VOLTS,  01.450  VOLTS

VIL\H @1.8v =  00.600  VOLTS,  01.050  VOLTS

VOL2 @4.5v =  00.040  VOLTS

VOL2 @3.0v =  00.050  VOLTS

VOL1 @1.8v =  00.000  VOLTS

IOL2 @4.5v =  01.01      MA limit!

IOL2 @3.0v =  13.56      MA

IOL1 @1.8v =  04.12      MA

tLOW, no test.

tHIGH (b,a) @5.5v = 0051.00 NS limit!, 0051.00 NS limit!

tHIGH (b,a) @5.0v = 0064.00 NS, 0065.00 NS

tHIGH (b,a) @4.5v = 0072.00 NS, 0073.00 NS

tHIGH (b,a) @3.6v = 0095.00 NS, 0096.00 NS

tHIGH (b,a) @2.7v = 0142.00 NS, 0142.00 NS

tHIGH (b,a) @2.5v = 0159.00 NS, 0160.00 NS

tHIGH (b,a) @1.8v = 0280.00 NS, 0281.00 NS

tI @5.5v = 0054.00 NS

tI @5.0v = 0061.00 NS

tI @4.5v = 0070.00 NS

tI @3.6v = 0093.00 NS

tI @2.7v = 0138.00 NS

tI @2.5v = 0155.00 NS

tI @1.8v = 0274.00 NS

tAA (b,a) @5.5v = 0223.00 NS, 0228.00 NS

tAA (b,a) @5.0v = 0236.00 NS, 0241.00 NS

tAA (b,a) @4.5v = 0251.00 NS, 0257.00 NS

tAA (b,a) @3.6v = 0294.00 NS, 0301.00 NS

tAA (b,a) @2.7v = 0380.00 NS, 0389.00 NS

tAA (b,a) @2.5v = 0413.00 NS, 0422.00 NS

tAA (b,a) @1.8v = 0657.00 NS, 0665.00 NS

tBUF @5.5v = 0059.00 NS

tBUF @5.0v = 0065.00 NS

tBUF @4.5v = 0076.00 NS

tBUF @3.6v = 0102.00 NS

tBUF @2.7v = 0152.00 NS

tBUF @2.5v = 0171.00 NS

tBUF @1.8v = 0302.00 NS

tHD.STA @5.5v = 4500 - 4468.00 NS

tHD.STA @5.0v = 4500 - 4466.00 NS

tHD.STA @4.5v = 4500 - 4463.00 NS

tHD.STA @3.6v = 4500 - 4455.00 NS

tHD.STA @2.7v = 4500 - 4437.00 NS

tHD.STA @2.5v = 4500 - 4429.00 NS

tHD.STA @1.8v = 4500 - 4378.00 NS

tSU.STA @5.5v = 0047.00 NS

tSU.STA @5.0v = 0054.00 NS

tSU.STA @4.5v = 0063.00 NS

tSU.STA @3.6v = 0086.00 NS

tSU.STA @2.7v = 0132.00 NS

tSU.STA @2.5v = 0149.00 NS

tSU.STA @1.8v = 0267.00 NS

tHD.DAT @5.5v = 4736.00 NS - 4750

tHD.DAT @5.0v = 4734.00 NS - 4750

tHD.DAT @4.5v = 4730.00 NS - 4750

tHD.DAT @3.6v = 4719.00 NS - 4750

tHD.DAT @2.7v = 4697.00 NS - 4750

tHD.DAT @2.5v = 4688.00 NS - 4750

tHD.DAT @1.8v = 4666.00 NS - 4750

tSU.DAT @5.5v = 1000 - 0969.00 NS

tSU.DAT @5.0v = 1000 - 0969.00 NS

tSU.DAT @4.5v = 1000 - 0969.00 NS

tSU.DAT @3.6v = 1000 - 0966.00 NS

tSU.DAT @2.7v = 1000 - 0959.00 NS

tSU.DAT @2.5v = 1000 - 0955.00 NS

tSU.DAT @1.8v = 1000 - 0918.00 NS

tSU.STO @5.5v = 1000 - 1011.00 NS

tSU.STO @5.0v = 1000 - 1015.00 NS

tSU.STO @4.5v = 1000 - 1018.00 NS

tSU.STO @3.6v = 1000 - 1026.00 NS

tSU.STO @2.7v = 1000 - 1039.00 NS

tSU.STO @2.5v = 1000 - 1045.00 NS

tSU.STO @1.8v = 1000 - 1082.00 NS

tDH = tAA.

tWR @5.5v =  01.35mS

tWR @5.0v =  01.35mS

tWR @4.5v =  01.35mS

tWR @3.6v =  01.50mS

tWR @2.7v =  01.65mS

tWR @2.5v =  01.65mS

tWR @1.8v =  01.80mS

----------------------------------------