Quality and Reliability Data

Last Updated on 7/10/2002

Atmel Serial EEPROM AT24C04

Description		4K 2-Wire Shrink

Entry Date		2/23/2001

Tapeout Date		2/07/2002

Fab Out Date		5/25/2002

Char Date		5/26/2002

Release Engineer	Neal Scheibe

Sustaining Engineer	Neal Scheibe

#### Package Options

PACKAGE_OPTION		FIT/ROUTE		WAIVER	      	COMMENTS

EIAJ 8LD                                  YES                                 	NO

###### PDIP 8LD 	YES	NO

SOIC 8LD                                 	YES                                 	NO

TSSOP 14LD      	YES                        	NO

###### DBGA                                        NO                                  	NO

LAP 5X6                                   	NO                                  	NO

LAP 5X8                                   	NO                                  	NO

TSSOP                                       YES                                	NO

##### Voltage Options

AUTOMOTIVE	INDUSTRIAL     	COMMERCIAL

VOLTAGE		GRADE		GRADE	GRADE

5.00				PENDING		YES			YES

2.70				PENDING		YES			YES

1.80           			PENDING		YES			YES

Write Endurance

TRACK #	TEST_POINT	FAIL_QTY	 COMMENTS

8441                  1000000	0	NO FAILS

##### Life Test

TRACK #	TEST_POINT	FAIL_QTY	 COMMENTS

8441                   504                   0            		NO FAILS

Data Retention

TRACK #	RD_POINT      FAIL_QTY	COMMENTS

8441     	504                    0 		   NO FAILS

Characterization Data

Characterization Program

CH24C04  Serial EEPROM  11-03-98

----------------------------------------

Opens:

Shorts:

Lkg @5.5V, #1 =  00.01      UA limit!

Lkg @5.5V, #2 =  00.01      UA limit!

Lkg @5.5V, #3 =  00.01      UA limit!

Lkg @5.5V, #5 =  00.03      UA

Lkg @5.5V, #6 =  00.01      UA limit!

Lkg @5.5V, #7 =  00.19      UA

ISB @ 06.000  VOLTS =  003.3      UA

ISB @ 05.500  VOLTS =  002.7      UA

ISB @ 05.000  VOLTS =  002.2      UA

ISB @ 04.500  VOLTS =  001.6      UA

ISB @ 04.000  VOLTS =  001.2      UA

ISB @ 03.500  VOLTS =  001.0      UA

ISB @ 03.000  VOLTS =  000.5      UA

ISB @ 02.500  VOLTS =  000.1      UA

ISB @ 02.000  VOLTS =  000.1      UA limit!

ISB @ 01.500  VOLTS =  000.1      UA

Ack & Slaves:

Ack LVcc = 01.200  VOLTS limit!

WR-RD byte LVcc = 01.360  VOLTS

WR-RD array LVcc = 01.460  VOLTS

WR/RD MATRIX AT 100KHz:

5.5v 5.0v 4.5v 3.6v 2.7v 2.5v 1.8v 1.7v

---- ---- ---- ---- ---- ---- ---- ----

wrCHK @5.5v, rd:  P    P    P    P    P    P    P    P

wrICK @5.5v, rd:  P    P    P    P    P    P    P    P

wrCHK @5.0v, rd:  P    P    P    P    P    P    P    P

wrICK @5.0v, rd:  P    P    P    P    P    P    P    P

wrCHK @4.5v, rd:  P    P    P    P    P    P    P    P

wrICK @4.5v, rd:  P    P    P    P    P    P    P    P

wrCHK @3.6v, rd:  P    P    P    P    P    P    P    P

wrICK @3.6v, rd:  P    P    P    P    P    P    P    P

wrCHK @2.7v, rd:  P    P    P    P    P    P    P    P

wrICK @2.7v, rd:  P    P    P    P    P    P    P    P

wrCHK @2.5v, rd:  P    P    P    P    P    P    P    P

wrICK @2.5v, rd:  P    P    P    P    P    P    P    P

wrCHK @1.8v, rd:  P    P    P    P    P    P    P    P

wrICK @1.8v, rd:  P    P    P    P    P    P    P    P

wrCHK @1.7v, rd:  P    P    P    P    P    P    P    P

wrICK @1.7v, rd:  P    P    P    P    P    P    P    P

WR/RD MATRIX AT 400KHz:

5.5v 5.0v 4.5v 3.6v 2.7v 2.5v 1.8v 1.7v

---- ---- ---- ---- ---- ---- ---- ----

wrCHK @5.5v, rd:  P    P    P    P    P    P    F    F

wrICK @5.5v, rd:  P    P    P    P    P    P    F    F

wrCHK @5.0v, rd:   P    P    P    P    P    P    F    F

wrICK @5.0v, rd:   P    P    P    P    P    P    F    F

wrCHK @4.5v, rd:   P    P    P    P    P    P    F    F

wrICK @4.5v, rd:   P    P    P    P    P    P    F    F

wrCHK @3.6v, rd:   P    P    P    P    P    P    F    F

wrICK @3.6v, rd:   P    P    P    P    P    P    F    F

wrCHK @2.7v, rd:   P    P    P    P    P    P    F    F

wrICK @2.7v, rd:   P    P    P    P    P    P    F    F

wrCHK @2.5v, rd:   P    P    P    P    P    P    F    F

wrICK @2.5v, rd:   P    P    P    P    P    P    F    F

wrCHK @1.8v, rd:   F    F    F    F    F    F    F    F

wrICK @1.8v, rd:   P    P    P    P    P    P    F    F

wrCHK @1.7v, rd:   F    F    F    F    F    F    F    F

wrICK @1.7v, rd:   P    P    P    P    P    P    F    F

WR/RD MATRIX AT 1MHz:

5.5v 5.0v 4.5v 3.6v 2.7v 2.5v 1.8v 1.7v

---- ---- ---- ---- ---- ---- ---- ----

wrCHK @5.5v, rd:   F    F    F    F    F    F    F    F

wrICK @5.5v, rd:   F    F    F    F    F    F    F    F

wrCHK @5.0v, rd:   F    F    F    F    F    F    F    F

wrICK @5.0v, rd:   F    F    F    F    F    F    F    F

wrCHK @4.5v, rd:   F    F    F    F    F    F    F    F

wrICK @4.5v, rd:   F    F    F    F    F    F    F    F

wrCHK @3.6v, rd:   F    F    F    F    F    F    F    F

wrICK @3.6v, rd:   F    F    F    F    F    F    F    F

wrCHK @2.7v, rd:   F    F    F    F    F    F    F    F

wrICK @2.7v, rd:   F    F    F    F    F    F    F    F

wrCHK @2.5v, rd:   F    F    F    F    F    F    F    F

wrICK @2.5v, rd:   F    F    F    F    F    F    F    F

wrCHK @1.8v, rd:   F    F    F    F    F    F    F    F

wrICK @1.8v, rd:   F    F    F    F    F    F    F    F

wrCHK @1.7v, rd:   F    F    F    F    F    F    F    F

wrICK @1.7v, rd:   F    F    F    F    F    F    F    F

fSCL@100KHz: Vcc = 01.360  VOLTS

fSCL@200KHz: Vcc = 01.380  VOLTS

fSCL@300KHz: Vcc = 01.480  VOLTS

fSCL@400KHz: Vcc = 01.700  VOLTS

fSCL@500KHz: Vcc = 01.940  VOLTS

fSCL@600KHz: Vcc = 02.200  VOLTS

fSCL@700KHz: Vcc = 02.460  VOLTS

fSCL@800KHz: Vcc = 02.700  VOLTS

fSCL@900KHz: Vcc = 03.020  VOLTS

fSCL@1.0MHz: Vcc = 03.420  VOLTS

fSCL@1.1MHz: Vcc = 04.060  VOLTS

fSCL@1.2MHz: Vcc = 05.400  VOLTS

fSCL@1.3MHz: Vcc = 06.000  VOLTS limit!

fSCL@1.4MHz: Vcc = 06.000  VOLTS limit!

fSCL@1.5MHz: Vcc = 06.000  VOLTS limit!

ICCrd @5.5v, 400KHz =  00.40      MA

ICCrd @5.0v, 400KHz =  00.30      MA

ICCrd @4.5v, 400KHz =  00.25      MA

ICCrd @3.6v, 100KHz =  00.15      MA

ICCrd @2.7v, 100KHz =  00.10      MA

ICCrd @2.5v, 100KHz =  00.05      MA limit!

ICCrd @1.8v, 100KHz =  00.05      MA limit!

ICCwr @5.5v, 100KHz =  03.45      MA

ICCwr @5.0v, 100KHz =  03.25      MA

ICCwr @4.5v, 100KHz =  03.70      MA

ICCwr @3.6v, 100KHz =  02.45      MA

ICCwr @2.7v, 100KHz =  01.60      MA

ICCwr @2.5v, 100KHz =  01.40      MA

ICCwr @1.8v, 100KHz =  00.45      MA

VIL\H @5.5v =  02.050  VOLTS,  02.750  VOLTS

VIL\H @5.0v =  01.900  VOLTS,  02.500  VOLTS

VIL\H @4.5v =  01.750  VOLTS,  02.250  VOLTS

VIL\H @3.6v =  01.450  VOLTS,  01.900  VOLTS

VIL\H @2.7v =  01.150  VOLTS,  01.500  VOLTS

VIL\H @2.5v =  01.050  VOLTS,  01.400  VOLTS

VIL\H @1.8v =  00.650  VOLTS,  01.000  VOLTS

VOL2 @4.5v =  00.050  VOLTS

VOL2 @3.0v =  00.060  VOLTS

VOL1 @1.8v =  00.000  VOLTS

IOL2 @4.5v =  01.01      MA limit!

IOL2 @3.0v =  11.98      MA

IOL1 @1.8v =  03.67      MA

tLOW, no test.

tHIGH (b,a) @5.5v = 0050.00 NS limit!, 0050.00 NS limit!

tHIGH (b,a) @5.0v = 0050.00 NS limit!, 0050.00 NS limit!

tHIGH (b,a) @4.5v = 0051.00 NS limit!, 0051.00 NS limit!

tHIGH (b,a) @3.6v = 0072.00 NS, 0072.00 NS

tHIGH (b,a) @2.7v = 0108.00 NS, 0108.00 NS

tHIGH (b,a) @2.5v = 0121.00 NS, 0122.00 NS

tHIGH (b,a) @1.8v = 0215.00 NS, 0219.00 NS

tI @5.5v = 0038.00 NS

tI @5.0v = 0044.00 NS

tI @4.5v = 0051.00 NS

tI @3.6v = 0069.00 NS

tI @2.7v = 0104.00 NS

tI @2.5v = 0118.00 NS

tI @1.8v = 0211.00 NS

tAA (b,a) @5.5v = 0314.00 NS, 0318.00 NS

tAA (b,a) @5.0v = 0323.00 NS, 0329.00 NS

tAA (b,a) @4.5v = 0338.00 NS, 0343.00 NS

tAA (b,a) @3.6v = 0371.00 NS, 0377.00 NS

tAA (b,a) @2.7v = 0437.00 NS, 0444.00 NS

tAA (b,a) @2.5v = 0464.00 NS, 0472.00 NS

tAA (b,a) @1.8v = 0645.00 NS, 0658.00 NS

tBUF @5.5v = 0050.00 NS limit!

tBUF @5.0v = 0053.00 NS

tBUF @4.5v = 0060.00 NS

tBUF @3.6v = 0085.00 NS

tBUF @2.7v = 0129.00 NS

tBUF @2.5v = 0145.00 NS

tBUF @1.8v = 0256.00 NS

tHD.STA @5.5v = 0011.00 NS limit!

tHD.STA @5.0v = 0011.00 NS limit!

tHD.STA @4.5v = 0011.00 NS limit!

tHD.STA @3.6v = 0011.00 NS limit!

tHD.STA @2.7v = 0011.00 NS limit!

tHD.STA @2.5v = 0011.00 NS limit!

tHD.STA @1.8v = 0011.00 NS limit!

tSU.STA @5.5v = 0030.00 NS

tSU.STA @5.0v = 0036.00 NS

tSU.STA @4.5v = 0041.00 NS

tSU.STA @3.6v = 0060.00 NS

tSU.STA @2.7v = 0096.00 NS

tSU.STA @2.5v = 0107.00 NS

tSU.STA @1.8v = 0201.00 NS

tHD.DAT @5.5v = 4735.00 NS - 4750

tHD.DAT @5.0v = 4733.00 NS - 4750

tHD.DAT @4.5v = 4731.00 NS - 4750

tHD.DAT @3.6v = 4724.00 NS - 4750

tHD.DAT @2.7v = 4707.00 NS - 4750

tHD.DAT @2.5v = 4701.00 NS - 4750

tHD.DAT @1.8v = 4662.00 NS - 4750

tSU.DAT @5.5v = 1000 - 0959.00 NS

tSU.DAT @5.0v = 1000 - 0958.00 NS

tSU.DAT @4.5v = 1000 - 0955.00 NS

tSU.DAT @3.6v = 1000 - 0950.00 NS

tSU.DAT @2.7v = 1000 - 0936.00 NS

tSU.DAT @2.5v = 1000 - 0930.00 NS

tSU.DAT @1.8v = 1000 - 0886.00 NS

tSU.STO @5.5v = 1000 - 1027.00 NS

tSU.STO @5.0v = 1000 - 1029.00 NS

tSU.STO @4.5v = 1000 - 1032.00 NS

tSU.STO @3.6v = 1000 - 1041.00 NS

tSU.STO @2.7v = 1000 - 1059.00 NS

tSU.STO @2.5v = 1000 - 1065.00 NS

tSU.STO @1.8v = 1000 - 1110.00 NS

tDH = tAA.

tWR@05.500  VOLTS =  01.20mS

tWR@05.400  VOLTS =  01.20mS

tWR@05.300  VOLTS =  01.20mS

tWR@05.200  VOLTS =  01.20mS

tWR@05.100  VOLTS =  01.20mS

tWR@05.000  VOLTS =  01.20mS

tWR@04.900  VOLTS =  01.20mS

tWR@04.800  VOLTS =  01.20mS

tWR@04.700  VOLTS =  01.20mS

tWR@04.600  VOLTS =  01.20mS

tWR@04.500  VOLTS =  01.20mS

tWR@04.400  VOLTS =  01.20mS

tWR@04.300  VOLTS =  01.20mS

tWR@04.200  VOLTS =  01.35mS

tWR@04.100  VOLTS =  01.35mS

tWR@04.000  VOLTS =  01.35mS

tWR@03.900  VOLTS =  01.35mS

tWR@03.800  VOLTS =  01.35mS

tWR@03.700  VOLTS =  01.35mS

tWR@03.600  VOLTS =  01.35mS

tWR@03.500  VOLTS =  01.35mS

tWR@03.400  VOLTS =  01.35mS

tWR@03.300  VOLTS =  01.35mS

tWR@03.200  VOLTS =  01.35mS

tWR@03.100  VOLTS =  01.35mS

tWR@03.000  VOLTS =  01.35mS

tWR@02.900  VOLTS =  01.35mS

tWR@02.800  VOLTS =  01.35mS

tWR@02.700  VOLTS =  01.35mS

tWR@02.600  VOLTS =  01.35mS

tWR@02.500  VOLTS =  01.50mS

tWR@02.400  VOLTS =  01.50mS

tWR@02.300  VOLTS =  01.50mS

tWR@02.200  VOLTS =  01.50mS

tWR@02.100  VOLTS =  01.50mS

tWR@02.000  VOLTS =  01.50mS

tWR@01.900  VOLTS =  01.65mS

tWR@01.800  VOLTS =  01.65mS

tWR@01.700  VOLTS =  01.65mS

tWR@01.600  VOLTS =  01.80mS

tWR@01.500  VOLTS =  01.95mS

----------------------------------------