Product Change Notification Number:   SC024001				Date: 	October 3, 2002

Title:  Marking revision indicators changed due to minor mask modification

Product Identification:

| AT29C512 | AT29LV512 |  |
| --- | --- | --- |
| AT29C010A | AT29LV010A | AT29BV010A |
| AT29C040A | AT29LV040A | AT29BV040A |

Reason for Change:	 Design	 Processing	 Logistics
	 Manufacturing Location	 Quality/Reliability	 Material

Change Description:

The marking revision indicator for the above listed parts has been changed from “A” to “B”.  The revision designator is located next to the date code on each device. This change is due to a minor modification to the silicon that rectified an anomaly that could cause improper programming when using the toggle bit to detect the end of a programming cycle.  If the toggle bit is not being used by the system to detect the end of a programming cycle, then this anomaly would not apply.

Identification Method to Distinguish Change:

The marking revision designator located next to the date code on each device will be will be changed from “A” to “B” for the affected products.  If the toggle bit is not being used, then customers can use either version.

Qualification Data:	 available	 will be available in WW___     	  not applicable

Samples:	 available	 will be available in WW44        not applicable

PPAP Notification:	 required	 not applicable

Quantifiable Impact on Quality & Reliability:

There is no impact on quality or reliability.

Estimated Implementation Date*:

Production shipments of the above listed products will commence by WW44.

*The Estimated Implementation Date is the forecasted date that a customer may expect to receive changed product.  This is determined by the estimated date of inventory depletion on the PCN issue date.  This may be affected by fluctuations in supply and demand.  Consequently, although customers should be prepared to receive changed product on this date, Atmel will continue to ship pre-changed product until a time in which inventory has been depleted.  This may result in pre-changed product being shipped to customers after this forecasted date.

Atmel Contact:	Quality	Marketing

| NAME | Pete Cannon |  | John Perlegos |
| --- | --- | --- | --- |
| PHONE: | (408) 436-4122 |  | (408) 487-2561 |
| FAX: | (408) 436-4284 |  | (408) 487-2799 |
| EMAIL: | pcannon@atmel.com |  | jlperlegos@atmel.com |

Atmel will deem this change accepted unless specific conditions of acceptance are provided in writing within 30 days from the date of this notice. All correspondence must be sent to the Quality Contact e-mail address listed above.

Information provided herein is in connection with Atmel products.  No license, express or implied, by estoppel or otherwise, to any intellectual property rights is granted by this document.  Except as provided in Atmel's Terms and Conditions of Sale for such products, Atmel assumes no liability whatsoever, and Atmel disclaims any express or implied warranty, including liability or warranties relating to fitness for a particular purpose, merchantability, or non-infringement of any patent, copyright or other intellectual property right.  Atmel products are not intended for use in a product or system intended to support or sustain life which, if it fails, can be reasonably expected to result in significant personal injury.  Atmel may make changes to specifications and product descriptions at any time, without notice.