GRAPHIC TYPE LCD MODULE

PRODUCT SPECIFICATION

# GPG11061WN1

Codico Ges.m.b.H

Mühlgasse 86-88

A-2380 Perchtoldsdorf

TEL :        +43 1 86 305 0  ,  FAX:      +43 1 86 305 98 0

1.0 INTRODUCTION

THIS SPEC MANUAL INTRODUCES THE OUTSIDE DIMENSIONS,                 OPTICAL CHARACTERISTICS, ELECTRICAL CHARACTERISTICS, INTERFACE SPECIFICATIONS, INSTRUCTIONS,ETC. OF THE CUSTOM DESIGN PRODUCT.

FEATURES OF THIS MODULE ARE LISTED AS BELOW：

(1)112 ×64 DOTS MATRIX LCD MODULE.

(2)FSTN LCD MODULE WITH 6 O‘CLOCK VIEWING DIRECTION.

(3)BACKLIGHT：NO BACKLIGHT.

(4)LSI DEVICES：KS0713H

(5)OUTLINE DIMENSION：36.6(W)×40.5(H)×2.0(D) mm

2.0 CLASSIFICATION OF MODULE

| GPG | 1106 | 1 W N |
| --- | --- | --- |

BACKLIGHT

N: NO BACKLIGHT (REFLECTIVE)

E: EL BACKLIGHT

B: BOTTOM LED

S: SIDE LED

BACKGROUND COLOR

T: TN TYPE LCD

W: (FSTN) BLACK & WHITE

G: (STN) GRAY

Y: (STN) YELLOW

B: (STN) BLUE

O: OTHER

VERSION NO (A TO J:COB ; K TO S: SMT

T TO Z: TAB ; 1 TO 9:COG )

PRODUCT NO.

SYMBOL OF GIANTPLUS‘S CUSTOMER

DESIGN LCD MODULE

3.0 MECHANICAL  SPECIFICATIONS

| ITEM | STANDARD VALUE | UNIT |
| --- | --- | --- |
| LCD TYPE | FSTN |  |
| MODULE DIMENSION | 36.6(W)×40.5(H)×2.0(D) | mm |
| VIEW AREA | 33.6(W)×20(H) | mm |
| DOT SIZE | 0.24(W)×0.27(H) | mm |
| DOT PITCH | 0.26(W)×0.29(H) | mm |
| DUTY | 1/65 |  |
| DISPLAY TYPE | 112×64 DOTS |  |
| VIEWING DIRECTION | 6 O‘CLOCK |  |

4.0 MAX. STANDARD VALUE

| ITEM | SYMBOL | MIN | MAX | UNIT |
| --- | --- | --- | --- | --- |
| OPERATING TEMPERATURE | Top | -20 | 70 | ℃ |
| STORAGE TEMPERATURE | Tst | -20 | 80 | ℃ |
| SUPPLY VOLTAGE FOR LOGIC | VDD-VSS |  | 7.0 | V |
| SUPPLY VOLTAGE FOR LCD | V0-VSS |  | 17 | V |
| LOGIC INPUT VOLTAGE | VI | 0 | VDD+0.3 | V |
| STATIC ELECTRICITY | BE SURE THAT USER ARE GROUNDED WHEN HANDLING LCM |  |  |  |

5.0 ELECTRICAL CHARACTERISTICS

| ITEM | SYMBOL |  | TEST CONDITION | MIN | TYP |  | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUPPLY VOLTAGE FOR LOGIC | VDD- VSS |  |  | 2.7 | 3.0 |  | 3.3 | V |
| SUPPLY VOLTAGE FOR LCD DRIVER | V0-VSS |  | Ta= 0℃ 25℃ 50℃ | 8.7 | 9.0 | 9.3 |  | V |
| INPUT VOLTAGE | VI | H |  | 0.8VDD |  | VDD |  | V |
| “H”＆“L”LEVEL |  | L |  | 0 |  | 0.2VDD |  |  |
| POWER SUPPLY CURRENT FOR VDD | IDD |  |  |  | 0.1 |  | 0.5 | mA |

6.0 OPTICAL CHARACTERISTICS

| ITEM | SYMBOL |  |  | CONDITION | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VIEWING | Θ |  |  | Cr≧2,Ta=25℃ | -20 |  | 40 | deg |
| ANGLE | Φ |  |  |  | -30 |  | 30 |  |
| CONTRAST |  | RATIO(MAX) | CR | Θ=0°Φ=0° |  | 6 |  |  |
| RESPONSE |  | TIME(RISE) | TR | Ta=25℃ |  |  | 170 | ms |
| RESPONSE |  | TIME(FALL) | TF |  |  |  | 210 | ms |

6.1 OPTICAL MEASUREMENT SYSTEM

MEASURING INSTRUMENT FOR ELECTRO-OPTICAL CHARACTERISTICS

normal

light source (*1)

photo-meter

Θ0  Θ         Θ0 - Θ = 30°

polarizer

LCD

(*2)

*1: LIGHT SOURCE POSITION FOR MEASURING OF REFLECTIVE TYPE LCD

*2: LIGHT SOURCE POSITION FOR MEASURING OF TRANSMISSIVE/

TRANSFLECTIVE TYPE LCD

6.2 DEFINITION OF    AND  

=0                                        =0

Θ>0

>0                      <0

Θ<0

6.3 DEFINITION OF CONTRAST RATIO CR

DEFINITION :

brightness of non-selection dot (bns)

Ｃｒ　＝

brightness of selection dot (bs)

BRIGHTNESS

Brightness curve of selection dot

Brightness curve of non-selection dot

Bns

Bs

DRIVING VOLTAGE  (V)

6.4 DEFINITION OF OPTICAL RESPONSE TIME

NON-SELECTED STATE      SELECTED STATE    NON-SELECTED

BRIGHTNESS                                                        90%

100%

10%

Tr                       Tf

7.0 INTERFACE PIN FUNCTION DESCRIPTION

| PIN NO | SYMBOL | FUNCTIONS |
| --- | --- | --- |
| 28-24 | V4-V0 | LCD BIAS VOLTAGE |
| 23 | C2- | CAPACITOR CONNECTOR FOR THE INTERNAL VOLTAGE CONVERTER |
| 22 | C2+ | CAPACITOR CONNECTOR FOR THE INTERNAL VOLTAGE CONVERTER |
| 21 | C1- | CAPACITOR CONNECTOR FOR THE INTERNAL VOLTAGE CONVERTER |
| 20 | C1+ | CAPACITOR CONNECTOR FOR THE INTERNAL VOLTAGE CONVERTER |
| 19 | C3- | CAPACITOR CONNECTOR FOR THE INTERNAL VOLTAGE CONVERTER |
| 18 | C3+ | CAPACITOR CONNECTOR FOR THE INTERNAL VOLTAGE CONVERTER |
| 17 | VOUT | VOLTAGE CONVERTER OUTPUT |
| 16 | VDD | POWER SUPPLY FOR LOGIC |
| 15 | VSS | GROUND |
| 14 | MI | 6800 OR 8080 SERIES |
| 13-6 | DB7-DB0 | DATA BIT |
| 5 | E | ENABLE |
| 4 | RW | READ/WRITE |
| 3 | RS | REGISTER SELECT INPUT |
| 2 | RESET | RESET |
| 1 | CS1B | CHIP SELECT INPUTS |

8.0 PRECAUTION IN APPLICATION

8.1 LCD DRIVING VOLTAGE AND VIEWING ANGLE RANGE

SETTING VHH OUT OF THE RECOMMENDED CONDITION WILL BE A CAUSE        FOR A CHANGE OF VIEWING ANGLE RANGE .

8.2 ELECTRO-STATIC DISCHARGE

AS THIS MODULE IS PROVIDED WITH C-MOS LSI, PRECAUTIONS MUST BE                TAKEN WHEN HANDLING IT.

8.3 HANDLING

(1) AS THE ADHESIVES USED FOR ADHERING UPPER/LOWER POLARIZERS AND ALUMINUM PLATES ARE MADE OF ORGANIC, SUBSTANCES WHICH WILL BE DETERIORATED BY A CHEMICAL REACTION WITH SUCH CHEMICALS AS ACETONE, TULUENE, ETHANOLE AND ISOPROPYLALCOHOL.

(2) IMMEDIATELY WIPE OFF SALIVA OR WATER DROP ATTACHED ON THE DISPLAY AREA BECAUSE ITS LONG PERIOD ADHERENCE MAY CAUSE DEFORMATION OR FADED COLOR ON THE SPOT.

(3) FOGY DEW DEPOSITED ON THE SURFACE AND CONTACT TERMINALS DUE TO COLDNESS WILL BE A CAUSE FOR POLARIZER DAMAGE, STAIN AND DIRT ON PRODUCT. WHEN NECESSARY TO TAKE OUT THE PRODUCTS FROM SOME PLACE AT LOW TEMPERATURE FOR TEST,ETC. IT IS REQUIRED FOR THEM TO BE WARMED UP IN A CONTAINER ONCE AT THE TEMPERATURE HIGHER THAN THAT OF ROOM.

(4) TOUCHING THE DISPLAY AREA AND CONTACT TERMINALS WITH BARE HANDS AND CONTAMINATING THEM ARE PROHIBITED, BECAUSE THE STAIN ON THE DISPLAY AREA AND POOR INSULATION BETWEEN TERMINALS ARE OFTEN CAUSED BY BEING TOUCHED BY BARE HANDS.( THERE ARE SOME COSMETICS DETRIMENTAL TO POLARIZERS.)

(5) IN GENERAL THE QUALITY OF GLASS IS FRAGILE THAT IT TENDS TO BE CRACKED OR CHIPPED IN HANDLING, SPECIALLY ON ITS PERIPHERY. PLEASE BE CAREFUL NOT TO GIVE IT SHARP SHOCK CAUSED BY DROPPING DOWN, ETC.

(6) RESPONSE TIME WILL BE EXTREMELY DELAYED AT LOWER

TEMPERATURE THAN THE OPERATING TEMPERATURE RANGE ON THE

OTHER HAND AT HIGHER TEMPERATURE LCD WILL SHOW DARK

BLUE COLOR IN THEM. HOWEVER THOSE PHENOMENA DO NOT MEAN

MALFUNCTION OR OUT OF ORDER WITH LCD WHENEVER COME BACK

IN THE SPECIFIED OPERATING TEMPERATURE RANGE.

(7) IF THE DISPLAY AREA IS PUSHED HARD DURING OPERATION, SOME

FONT WILL BE ABNORMALLY DISPLAYED BUT IT RESUMES NORMAL

CONDITION AFTER TUREING OFF ONCE.

8.4.STORAGE

(1) DON‘T LEAVE THE PRODUCT IN THE PLACE OF HIGH HUMIDITY FOR A

LONG PERIOD OF TIME. FOR THEIR STORAGE IN THE PLACE WHERE

TEMPERATURE IS 35℃ OR HIGHER, SPECIAL CARE TO PREVENT

THEMFROM HIGH HUMIDITY IS REQUIRED. A COMBINATION OF HIGH

TEMPERATURE AND HIGH HUMIDITY MAY CAUSE DEGRADATED AS

WELL AS BUBBLE GENERATED AND POLARIZER PEEL-OFF. THE

RELATIVE CONDITION OF 40℃ 50％ RH OR LESS IS RECOMMENDED.

(2)IN CASE OF STORING FOR A LONG PERIOD OF TIME (FOR INSTANCE,

FOR YEARS) FOR THE PURPOSE OF REPLACEMENT USING, THE

FOLLOWING WAYS ARE RECOMMENDED

(2.1) STORAGE IN A POLYETHYLENE BAG WITH OPENING SEALED SO

AS NOT TO ENTER FRESH AIR OUTSIDE IN IT, AND WITH NO

DESICCANT.

(2.2) PLACING IN A DARK PLACE WHERE NEITHER EXPOSURE TO

DIRECT SUNLIGHT NOR LIGHT, KEEPING TEMPERATURE IN THE

RANGE FROM 0℃ TO 35℃.

8.5 SAFTY

(1) WHEN ANY LIQUID LEAKED OUT OF A DAMAGED GLASS CELL

COMES IN CONTACT WITH YOUR HANDS, PLEASE WASH IT OFF

WELL WITH SOAP AND WATER.

(2) IT IS RECOMMENDABLE TO CRASH DAMAGED OR UNECESSARY LCD‘s

INTO PIECES AND WASH OFF LIQUID CRYSTAL BY EITHER OF

SOLVENTS SUCH AS ACETONE OR ETHANAL, WHILE SHOULD BE

BURNED UP LATER.

9.0 COSMETIC CRITERIA

9.1 LCD VISUAL & ELECTRONIC INSPECTION STANDARD

A:Fluorelamp:20W;Viewing distance:30cm. B:Viewing Angle:NORMAL

NOMAL

20W Power                                                     Upper polarizer

Lamp

Glass

Lower polarizer

9.2 SCREEN COSMETIC CRITERIA

| NO. | DEFECT | JUDGEMENT CRITERION |  |
| --- | --- | --- | --- |
| 1 | Including black and whitespots, pin holes and which must be within one pixel size | Size:d mm | Acceptable Qty in activearea |
|  |  | d<0.1 0.1<d<0.2 0.2<d<0.3 0.3<d | Disregard 5 3 0 |
|  |  | Note:d=(Length+Width)/2 |  |
| 2 | Line | L 5.0 2.0 (4) See No.1 0.03 0.05 0.1 W Note:( ):Acceptable Qty L :Length (mm) W:Width(mm) :Disregard |  |
| 3 | Bubbles in polarizer | Size:d mm | Acceptable Qty in activearea |
|  |  | d<0.3 0.3<d<0.6 0.6<d<1.0 1.0<d | Disregard 3 1 0 |

NO         Defect                   Judgement  Criterion

4     Dot matrix                 Size:d mm     Acceptable Qty in active area

Pin Hole

Lack        d≦0.1            Disregard

0.1＜d≦0.2                5

0.2＜d≦0.3                3

0.3＜d                     0

Note:d=(Length＋Width)/2

5

A             1. A:Distortion seqare ≦  ±15％   Accept

B     2. B:Distortion seqare ≦  ±15％   Accept

10.0 OUTLINE DIMENSION